Physiological Variables for Pain Quantification Device

1. Heart Rate (HR)

Heart rate is calculated from the time interval between two consecutive heartbeats (RR interval).

Formula:

HR = 60 / RR

Where:

HR = Heart rate (beats per minute)

RR = Time interval between two heartbeats (seconds)

Example:

If RR = 0.8 s

HR = 60 / 0.8 = 75 bpm

2. Heart Rate Variability (HRV)

HRV represents the variation between consecutive heartbeats.

Standard Deviation of NN intervals (SDNN):

SDNN = sqrt( (1/(N-1)) * Σ(RRi - RRmean)^2 )

Where:

RRi = Individual RR interval

RRmean = Mean RR interval

N = Number of intervals

Lower HRV is often associated with higher stress and pain levels.

3. Skin Temperature Change

Pain and inflammation can cause localized temperature changes.

ΔT = Tlocal − Tbaseline

Where:

ΔT = Temperature difference

Tlocal = Measured skin temperature

Tbaseline = Baseline skin temperature

Example:

ΔT = 37.8 − 36.5 = 1.3 °C

4. Galvanic Skin Response (GSR)

Skin conductance increases with sympathetic nervous system activation.

GSR = 1 / R

Where:

GSR = Skin conductance (Siemens)

R = Skin resistance (Ohms)

Pain increases sweating, which decreases resistance and increases conductance.

5. Electromyography (EMG)

Muscle activity is quantified using the Root Mean Square (RMS) value of the EMG signal.

EMG_RMS = sqrt( (1/N) * Σ(xi^2) )

Where:

xi = EMG signal amplitude

N = Number of samples

Higher RMS values may indicate muscle guarding or spasm associated with pain.

6. Electroencephalography (EEG) Power

EEG activity related to pain can be analyzed using Power Spectral Density (PSD).

P(f) = |X(f)|^2

Where:

X(f) = Fourier transform of the EEG signal

P(f) = Power at frequency f

Pain-related activity often involves changes in:

Theta band (4–8 Hz)

Beta band (13–30 Hz)

Gamma band (>30 Hz)